Optisweep Phase 0 — Issue Category Reference

UPS Haslet, TX · Data sources: Salesforce RCA spreadsheet, Confluence KBAs, Teams RCA Chat, RCA PDF

Generated: May 2026 · 30 cases analyzed · 4 confirmed categories

# Category Summary

| Category | Description | Cases | Priority | Entry symptom | Resolution approach | Support-safe? |
| --- | --- | --- | --- | --- | --- | --- |
| CAT-1 | WCS/Service Failure | 7 cases | P1/P2 | AGVs stopped, no alarms, tipper heartbeat timeout | Service restart (Optisweep/Ignition), E-stop sequence | Partial — E-stop site-safe; service restart requires engineer remote access |
| CAT-2 | AGV State Mismatch | 11 cases | P2 | Robot wrong zone, unmapped, stuck in bag-out/staging/shutdown | Remove/re-add specific AGVs via RMS | Partial — most steps RMS-accessible; full shuttle rebuild requires engineer |
| CAT-3 | Tote Record Mismatch | 5 cases | P2/P3 | Ghost tote, bad tote ID, tote stuck in RMS after removal | Cancel tasks in WCS Task Management, remove/re-add tote | Yes — Task Management is support-executable |
| CAT-4 | Power/Server Recovery | 5 cases | P1/P2 | Post-power-outage or server-unresponsive state | Restore power, reboot server, start VMs, rebuild tote data | No — escalate immediately; requires engineer and Infrastructure team |

# CAT-1: WCS/Service Failure — 7 cases

Entry symptom: AGVs stopped with no active alarms. Tipper heartbeat timeout or Ignition/Optisweep service down. Resolution requires engineer remote access for service restart.

| incident_id | source | issue_category | symptom_summary | component | root_cause | resolution_steps | escalation_trigger | resolution_status | site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 229374 | Salesforce RCA + Teams RCA Chat + RCA PDF | CAT-1: WCS/Service Failure | AGVs stopped mid-sort. No active alarms on RMS. All tippers showing heartbeat timeout, status 'active'. Hospital tote removal hangs. Beacon lights green at operator stations. | WCS/Ignition, Tipper OTnet endpoint (10.27.80.157), AGV fleet | WCS/Ignition lost stable OPC-UA communication with tipper OTnet endpoint. Most likely triggered by OT-side controls device failure (RIO Module Ethernet Failure, ClearLink Module not communicating, VFD 1/2 not detected). DB timeouts were secondary symptoms. | 1. Confirm all beacon lights at operator stations are green. 2. Verify no active hardware alarms on RMS screen. 3. Have site place master E-stop ON. 4. [Engineer] Restart Optisweep WCS/Ignition service remotely. 5. [Engineer] Pause then unpause all tippers via Ignition. 6. Verify tipper heartbeat values return to non-zero and updating. 7. Confirm AGVs resume movement and hospital tote removal executes. | Remote access unavailable (VPN/ZScaler outage). Hardware alarms present (RIO Module, VFD, ClearLink). Heartbeat does not recover after service restart. Issue recurs within same sort window. | Resolved — 6.5 hours (delayed by ZScaler VPN outage) | UPS Haslet, TX |
| 229716 | Salesforce RCA spreadsheet | CAT-1: WCS/Service Failure | Sort started fine. As soon as AGVs lined up to go to tippers, all 3 lines stopped. Could not remove tote at hospital — command would not execute. All tippers showing heartbeat timeout, status active. No faults on RMS screen. | WCS/Ignition, Tipper heartbeat, AGV fleet, hospital station | Tipper heartbeat failure. Same pattern as 229374. Exact OT-side initiating cause not re-isolated. | 1. Confirm site ready and sort can be paused. 2. Have site place master E-stop ON. 3. [Engineer] E-stop system and restart Optisweep service. 4. [Engineer] Confirm AGV operation and hospital function restored. 5. [Engineer] Download Server Event and Ignition logs for review. | Heartbeat does not recover after restart. OT hardware alarms present. Remote access unavailable. Recurrence within same shift. | Resolved — same day | UPS Haslet, TX |
| 229777 | Salesforce RCA spreadsheet | CAT-1: WCS/Service Failure | System put to startup, everything waiting, no path shown, nothing reaching tippers. Same presentation as 229374 and 229716. | WCS/Ignition, Tipper heartbeat, AGV fleet | Tipper heartbeat failure — confirmed recurrence of same pattern as 229716. | 1. Have site place master E-stop ON. 2. [Engineer] Restart Optisweep service remotely. 3. [Engineer] Pause then unpause tippers. 4. Confirm AGVs resume movement. 5. Confirm hospital function restored. | Heartbeat does not recover after restart. Hardware alarms present. Remote access unavailable. | Resolved — same day | UPS Haslet, TX |
| 229488 | Salesforce RCA spreadsheet | CAT-1: WCS/Service Failure | AGVs not moving. No fault shown. | Optisweep Windows service, AGV fleet | Optisweep Windows service stopped or became unresponsive. Specific trigger not investigated. | 1. Confirm area is stopped and safe to restart. 2. [Engineer] Restart Optisweep Windows service. 3. Confirm AGVs resume normal operation. | AGVs remain stopped after service restart. Active hardware alarms present. | Resolved — same day | UPS Haslet, TX |
| 228086 | Salesforce RCA spreadsheet | CAT-1: WCS/Service Failure | AGVs not moving. No fault shown. System showing standard operation mode. Web application faulted at 6:36 AM. | Optisweep Windows service, WCS web application, AGV fleet | Web application faulted due to memory usage. Service crash caused AGVs to lose sync. Infrastructure follow-up required to downsize logs. | 1. [Engineer] Restart Optisweep Windows service. 2. [Engineer] Open RMS and identify AGVs in incorrect/out-of-sync state. 3. [Engineer] Correct each out-of-sync AGV one by one in RMS. 4. Site resumes auto bag-out procedure to complete the sort. | Memory usage root cause unresolved — requires Infrastructure team. Service crashes repeatedly. AGV states cannot be corrected via RMS without engineer access. | Resolved — same day. Infrastructure follow-up required for log sizing. | UPS Haslet, TX |
| 223554 | Confluence KBA | CAT-1: WCS/Service Failure | Robot unable to return to normal operation at ACB Hospital Station. Robot in shutdown state. Unable to induct totes. | Optisweep services, ACB Hospital Station, AGV/robot | Robot in shutdown state. Root cause tied to prior case. Service reset required to clear state. | 1. [Engineer] Reset Optisweep services. 2. [Engineer] Perform site restart of the Hospital Lane. 3. Confirm robot returns to normal operation. 4. Confirm tote induction resumes without errors. | Robot does not return to normal operation after service reset and hospital lane restart. | Resolved | UPS Haslet, TX |
| 227895 | Salesforce RCA spreadsheet | CAT-1: WCS/Service Failure | Unable to remove tote via scanner or HMI. Scanner reads and says 'removing tote from system' but does not complete. Unable to initialize any tote. Ignition had crashed. | Ignition/WCS service, hospital tote management, AGV fleet | Ignition crashed. AGVs stuck in incorrect move states. Zone missing required AGV count. Startup AGVs failed to leave charger. | 1. [Engineer] Restart Ignition/WCS services. 2. [Engineer] In RMS, identify blue-circle AGVs stuck at stations. 3. [Engineer] Execute task for each stuck AGV. 4. [Engineer] Remove and re-add green-state AGVs as needed. 5. [Engineer] Disable chargers temporarily to force stuck charging AGVs to leave. 6. [Engineer] Verify each zone has required AGV count. 7. Confirm tote removal and initialization resume normally. | Ignition crash repeats. Zone AGV count cannot be corrected. Charger disable does not clear stuck AGVs. Tote removal still fails after service restart. | Resolved — 3 days (Feb 15-18) | UPS Haslet, TX |

# CAT-2: AGV State Mismatch — 11 cases

Entry symptom: System stuck in bag-out, staging, or shutdown. Robot unmapped, wrong zone, or lost task. Fix is remove/re-add specific AGVs via RMS. Most steps support-accessible.

| incident_id | source | issue_category | symptom_summary | component | root_cause | resolution_steps | escalation_trigger | resolution_status | site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 228723 | Salesforce RCA spreadsheet | CAT-2: AGV State Mismatch | Zone 5 unable to get a pair. AMR had low battery under rack. After AMR replacement, every AMR routes to hospital. | AGV fleet, RMS robot mapping | Robot 158 in the system but not mapped. Unmapped robot caused system-wide routing disruption. | 1. In RMS, identify robots shown as unmapped or with errors. 2. Remove the unmapped robot from the system via RMS. 3. Place removed robot on charger. 4. Monitor system for 10 minutes to confirm normal routing resumes. | System does not recover after robot removal. Multiple unmapped robots. Root cause of why robot became unmapped is unknown and recurs. | Resolved — same day | UPS Haslet, TX |
| 229589 | Salesforce RCA spreadsheet | CAT-2: AGV State Mismatch | System stuck in Bag Out mode. Cannot proceed to shutdown. System does not advance beyond Bag Out state. | AGV fleet, bag-out workflow, HMI | AGVs placed in incorrect positions (bots 186 and another). System cannot advance bag-out until all AGVs are in correct sequence. | 1. In RMS, identify AGVs in incorrect positions. 2. Remove out-of-position AGVs from the system. 3. Confirm remaining AGVs move into correct positions. 4. Re-add removed AGVs once positions are clear. 5. Confirm bag-out completes and system can proceed to shutdown. | System remains stuck in bag-out after AGV correction. Cannot identify which AGVs are incorrectly placed without engineer guidance. | Resolved — same day | UPS Haslet, TX |
| 221808 | Confluence KBA | CAT-2: AGV State Mismatch | Totes and robots out of sequence. Operational disruption. | AGV fleet, robot sequencing, WCS | Robot 2190 in incorrect state causing tote-robot sequencing failure. Software root cause not specified. | 1. Access control system for the robot. 2. Locate robot 2190 in RMS. 3. Remove robot 2190 from the system. 4. Re-add robot 2190 to the system. 5. Monitor tote and robot sequence to confirm proper alignment. | Sequencing issue persists after robot remove/re-add. Multiple robots affected. | Resolved | UPS Haslet, TX |
| 223326 | Confluence KBA | CAT-2: AGV State Mismatch | ACB AGVs not bagging. Emergency breakdown. Two robots absent from different zones. | AGV fleet, zone assignment, bagging workflow | Two robots absent from different zones, preventing bagging operations. | 1. Verify robot count assigned to each zone in RMS. 2. Identify missing robots and their last known location. 3. Manually force a bag-out operation to allow reassignment. 4. Reassign robots to the appropriate zones. 5. Confirm AGVs resume bagging without interruption. | Cannot identify missing robots via RMS. Force bag-out does not clear the issue. Zone robot counts cannot be corrected without engineer access. | Resolved | UPS Haslet, TX |
| 218656 | Confluence KBA | CAT-2: AGV State Mismatch | Single AGV (15-42159) not responding. Operational disruption. | Individual AGV, RMS, task assignment | AGV lost its task, resulting in non-responsive state. | 1. Wait until end of current sorting process. 2. Shut down the non-responsive AGV. 3. Reintegrate AGV into the system via RMS. 4. Confirm AGV responds to commands and resumes tasks. | AGV remains unresponsive after shutdown and reintegration. Issue affects multiple AGVs simultaneously. | Resolved | UPS Haslet, TX |
| 219465 | Confluence KBA | CAT-2: AGV State Mismatch | Multiple AGVs down. Sorter cannot transition to bag-out mode. | AGV fleet (all 23 shuttles), RMS, sorter | Software issue affecting AGVs and sorter functionality. Full shuttle rebuild required. | 1. [Engineer] Stop the RMS system. 2. [Engineer] Remove all 23 shuttles from the system. 3. [Engineer] Re-add all 23 shuttles to the system. 4. Confirm sorter can enter bag-out mode. 5. Monitor AGV and sorter operation. | Sorter still cannot enter bag-out after full shuttle rebuild. RMS will not stop or restart cleanly. | Resolved | UPS Haslet, TX |
| 219136 | Confluence KBA | CAT-2: AGV State Mismatch | Site stuck in Close Out Staging mode. Cannot transition to normal operations. | AGV fleet (AGV 191, AGV 155), zone assignment, HMI state | Issues with AGV 191 and AGV 155. Zone missing a required AGV. | 1. Identify and correct issues with AGV 191 in RMS. 2. Address issues with AGV 155. 3. Re-add AGV 155 to the zone that was missing an AGV. 4. Verify system transitions: Close Out Staging -> Bag Out -> Running. | System remains in Close Out Staging after AGV corrections. Cannot identify root issue with specific AGVs without engineer access. | Resolved | UPS Haslet, TX |
| 219684 | Confluence KBA | CAT-2: AGV State Mismatch | Unable to bag out from HMI. Bagging process will not complete. | AGV fleet, HMI, bag-out workflow | Robot placed in wrong spot, preventing bag-out completion. | 1. In RMS, identify the faulted or incorrectly placed AGV. 2. Remove the faulted AGV from the system. 3. Re-add the AGV to the system. 4. Confirm AGV is placed in correct location. 5. Test bag-out process from HMI. | Bag-out still blocked after AGV remove/re-add. Cannot identify faulted AGV via HMI without engineer guidance. | Resolved | UPS Haslet, TX |
| 218550 | Confluence KBA | CAT-2: AGV State Mismatch | System stopped at bag-out after sorting. System will not advance. | AGV fleet, bag-out workflow, exchange pair | AGV removed from grid without a tote, disrupting exchange pair sequence. | 1. Identify any AGV on the grid without a tote. 2. Remove that AGV from the grid. 3. Replace with a functional AGV. 4. Confirm all AGVs are in proper sequence. 5. Verify bag-out completes and system resumes normal operation. | System remains stuck in bag-out after AGV replacement. Cannot identify no-tote AGV without engineer access to grid view. | Resolved | UPS Haslet, TX |
| 219068 | Confluence KBA | CAT-2: AGV State Mismatch | Robots stalled and ceased movement during site shutdown. | AGV fleet, shutdown sequence | AGV out of place during shutdown sequence caused stall. | 1. Identify the out-of-place AGV. 2. Manually guide the AGV back to its designated destination. 3. Confirm all robots resume movement. 4. Monitor for further stalls. | Multiple AGVs stalled simultaneously. Manual guidance not possible remotely. | Resolved | UPS Haslet, TX |
| 223475 | Confluence KBA | CAT-2: AGV State Mismatch | Operational disruptions after AGV believed removed in wrong state. | AGV fleet, system state | AGV removed from system in incorrect state, causing system instability. | 1. Verify state of all AGVs in RMS. 2. Ensure all AGVs are adequately charged. 3. Place system in shutdown mode. 4. Restart system once all AGV checks are complete. 5. Monitor next sort cycle for further issues. | System does not stabilize after shutdown/restart. AGV state cannot be verified via RMS without engineer access. | Resolved | UPS Haslet, TX |
| 219158 | Salesforce RCA | . | ARBs not charging — stuck in shutdown |  | Robot missing from pair group. Pair group configuration requires engineer knowledge. |  |  |  |  |

# CAT-3: Tote Record Mismatch — 5 cases

Entry symptom: Tote stuck or phantom record in RMS. Bad tote ID from scanner, improper removal sequence, or ghost bin. Resolution via WCS Task Management — support-safe.

| incident_id | source | issue_category | symptom_summary | component | root_cause | resolution_steps | escalation_trigger | resolution_status | site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 220884 | Confluence KBA | CAT-3: Tote Record Mismatch | Tote 57 not removed from RMS after removal from hospital screen. Tote persists in system despite physical removal. | RMS, WCS Task Management, barcode scanner, tote ID | Padded tote ID caused mismatch between hospital hand scanner and system ID. Task stuck in system due to improper removal order. | 1. Navigate to Task Management in WCS. 2. Locate tasks associated with the stuck tote ID. 3. Cancel all tasks associated with that tote. 4. Remove the tote from the system via WCS. 5. Confirm tote no longer appears in RMS and tasks are cleared. | Tasks cannot be cancelled via Task Management. Multiple totes affected simultaneously. Tote ID scanning issue is systemic. | Resolved | UPS Haslet, TX |
| 230250 | Salesforce RCA spreadsheet | CAT-3: Tote Record Mismatch | Tote shows in RMS as held by robot with question mark. No physical tote present at rack location. Cannot induct to that spot. | RMS, tote tracking, rack | Ghost bin / phantom tote record in RMS. Exact initiating cause not determined. | 1. Attempt to induct a tote to the blocked or adjacent empty spot. 2. If ghost tote clears, continue normal operation. 3. If ghost tote persists, navigate to Task Management in WCS. 4. Cancel tasks associated with the ghost tote ID. 5. Remove ghost tote from system. | Ghost tote cannot be cleared via re-induction or WCS Task Management. Multiple ghost totes present. Recurs on same rack spot. | Resolved — workaround; root cause not confirmed | UPS Haslet, TX |
| 218343 | Confluence KBA | CAT-3: Tote Record Mismatch | System stuck in startup mode. WCS and RMS out of sync. System unresponsive during startup. | WCS, RMS, tote task management | Tote removed from system while in shutdown queue causing WCS-RMS synchronization failure. Physical state no longer matched system state. | 1. Verify physical tote layout on the floor. 2. [Engineer] In WCS, update system to reflect current physical tote state. 3. [Engineer] Re-sync WCS and RMS. 4. Restart system to initiate normal startup. 5. Confirm startup completes without error. | WCS-RMS re-sync fails. System remains stuck in startup after restart. Cannot determine current physical tote layout. | Resolved | UPS Haslet, TX |
| 218291 | Confluence KBA | CAT-3: Tote Record Mismatch | No totes available in AMR/AGV system for lane 3446 at station 462. No tasks generated for 24 hours. | RMS, lane/station assignment, tote inventory | Lane 3446 had no tote assigned to station 462, resulting in no tasks for 24 hours. | 1. Verify tote assignments for the affected lane and station in RMS. 2. Add a tote to the system for the missing lane/station. 3. Confirm tasks begin generating for that lane. 4. Monitor for 24 hours to ensure continued task processing. | Cannot add tote via RMS without engineer access. Issue affects multiple lanes/stations. | Resolved | UPS Haslet, TX |
| General_01 | Confluence General (Mar 23 2025 incident note) | CAT-3: Tote Record Mismatch | Tote removal stuck, stopping robot from moving. RMS removal attempt failed. Tote 63 still showing at induct despite physical removal. | WCS, RMS, hospital controller (AVEVA), barcode scanner | Tote scanned as 391063 instead of starting with '0'. WCS only checks that string value and barcode match — does not validate tote ID format. Incorrect scan inducted the tote under wrong ID. | 1. Attempt tote removal via hospital controller (AVEVA) first. 2. If tote persists in system, remove via WCS directly. 3. Do not attempt RMS removal until WCS/hospital controller steps are exhausted. 4. Confirm tote no longer showing at induct. | WCS and hospital controller removal both fail. Tote ID scanning issue is recurring across multiple totes. | Resolved via WCS removal | UPS Haslet, TX |

# CAT-4: Power/Server Recovery — 5 cases

Entry symptom: Post-power-outage or server-unresponsive state. AGVs or HMI non-functional. Escalate immediately — requires engineer and Infrastructure team involvement.

| incident_id | source | issue_category | symptom_summary | component | root_cause | resolution_steps | escalation_trigger | resolution_status | site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 219780 | Confluence KBA | CAT-4: Power/Server Recovery | Site experienced power outage. Optisweep system down. HMIs restored but AGVs remain non-operational. | Power supply, HMIs, AGV fleet, server infrastructure | Power outage affecting building power and network systems caused AGV malfunction. HMIs recovered on power restoration but AGVs did not auto-recover. | 1. Verify power supply to site is fully restored. 2. Check HMI status — confirm HMIs are operational. 3. [Engineer] Restart AGVs to bring them back online. 4. Monitor system for further issues or errors. | Power not fully restored. AGVs do not recover after restart attempt. Server infrastructure damaged. Network systems affected. | Resolved | UPS Haslet, TX |
| 219787 | Confluence KBA | CAT-4: Power/Server Recovery | HMI showing as operational but not providing state information after power outage. Date not displaying correctly. | HMI, Optisweep state, AGV tasks | Power outage caused HMI and Optisweep to lose state. System functional but state information not populating. | 1. Review power supply to HMI and Optisweep system — confirm all systems have power. 2. [Engineer] Re-add tasks for the troubled AGV to restore functionality. 3. Check HMI for additional error messages. 4. Confirm Optisweep is providing correct state information. | State information does not restore after task re-add. HMI errors persist. Power not confirmed stable. | Resolved | UPS Haslet, TX |
| 220256 | Confluence KBA | CAT-4: Power/Server Recovery | Totes not clearing off OptiSweep after power outage. Tasks paused. AGVs holding totes but not completing or releasing them. | AGV fleet, tote task management, WCS | Power outage caused tasks to pause. System unable to auto-resume tote clearing after power restore. | 1. [Engineer] Systematically remove totes from AGVs via RMS (MDI support required). 2. [Engineer] Re-add totes individually to AGVs. 3. Confirm tasks resume and totes clear normally. Note: This procedure currently requires engineer support — not validated as support-safe. | Engage engineer/MDI support immediately. Power outage is the trigger — treat as engineer-required until validated procedure established. | Resolved with MDI support | UPS Haslet, TX |
| 218971 | Confluence KBA | CAT-4: Power/Server Recovery | AGVs not starting despite showing as 'running' on HMI. Servers unresponsive. | Server infrastructure (ESX1, VMs), HMI, AGV fleet, network/communication | Communication/network issue caused servers to become unresponsive, preventing AGVs from starting despite HMI showing running status. | 1. Ping servers to verify responsiveness. 2. [Engineer] If servers do not respond, perform server reboot. 3. [Engineer] Log into ESX1 and start Virtual Machines. 4. Confirm RMS interface becomes accessible. 5. Confirm AGVs can be turned on and are operational. | Server reboot does not restore responsiveness. VMs will not start. Network connectivity cannot be confirmed. Infrastructure team engagement required. | Resolved | UPS Haslet, TX |
| 220078 | Confluence KBA | CAT-4: Power/Server Recovery | Optisweep robots not responding during test sort. Tasks showing as paused. System state blank. | Server, Optisweep services, AGV fleet, tote data | System state blank following server restart. All tote data lost. Robots unable to operate without valid tote configuration. | 1. [Engineer] Restart server and all related services per Server Restart KBA. 2. [Engineer] Remove all totes from system due to lost tote data. 3. [Engineer] Re-add all totes back into the system. 4. Confirm Optisweep robots are operational and responding during test sort. | Server restart does not restore state. Tote data cannot be re-added. Robots remain unresponsive after full procedure. | Resolved — all tote data lost and required reconfiguration | UPS Haslet, TX |

# Out of Scope — Phase 0

| Case | Issue | Reason excluded | Recommended handling |
| --- | --- | --- | --- |
| 229743 | E-Stop Zone Failure (zones 3/4/5) | Physical hardware — Branch PLC network cable reseating required. Cannot diagnose or resolve remotely. | Escalate immediately to controls engineer. Not a remote-supportable case. |
| 208967 | Wynright Auto Bagger down | Third-party equipment (Diafuku/Wynright). FORTNA resolution is contact vendor. | Direct customer to contact Diafuku/Wynright support directly. |
| 219158 | ARBs not charging — stuck in shutdown | Robot missing from pair group. Pair group configuration requires engineer knowledge. | Escalate to engineer. Flag for potential future CAT-2 sub-type if procedure validated. |
| 219086 | ACB stuck in shutdown (operator error) | Operator error root cause. Resolution is operator coaching, not system fix. | Walk operator through correct restart procedure. Document as training gap. |

Validation required from Wesley Saunders (controls engineer): (1) E-stop authority — can site operators execute independently? (2) RMS access level — which CAT-2 steps are available at operator vs engineer login? (3) CAT-4 tote rebuild procedure — can it be validated as support-safe with a written SOP?